# Computer Architecture Project Status Report

Custom log2 and sqrt ISA Extension for RV32IM with FPGA Acceleration and VGA Performance Visualization

# Scope of the Problem

In standard RV32IM architecture, integer square root (sqrt) and fixed-point logarithmic 2 are computed in software using iterative algorithms like Newton-Raphson. The iterative approach requires 80-150 clock cycles depending on the iteration count and specific multiplication and division algorithm in the hardware.

In real-world applications that require high throughput and low latency,  many heavy computations are completed in hardware using dedicated computational modules.

This project proposes:

- Extending a standard RV32IM core with custom hardware instructions:

  - sqrt – integer square root

  - log2 – integer based-2 logarithm

- Implementing hardware accelerator

- Measuring and comparing computation cycle counts between

  - Software implementation (combination of RV32IM multiplication and division, and addition conditional branches

  - Custom ISA instruction implementation

- Optionally visualizing results and performance on a VGA monitor

# GitHub Project Repository

https://github.com/yzhou276/Computer-Architecture-Project

# Project Progress

## RSIC-V R-Type Instructions & Encoding

- log2 	rd, rs1

- sqrt 	rd, rs1

Instruction Semantics

log2 	rd, rs1

x[rd] = floor(log2(x[rs1])) for rs1 != 0

sqrt 	rd, rs1

x[rd] = floor(sqrt(x[rs1]))

Two custom R-type instructions will be added to the existing RISC-V design. However, getting a high-level language compiler to generate assembly that uses these new instructions, or modifying the assembler/compiler flow so custom RISC-V assembly can be translated into machine code, would require compiler changes. That is really a separate area of computer engineering. I may explore how compilers work after this course, but for this project my focus is narrower: I will manually encode the log2 and sqrt instructions into machine code and load them onto the FPGA. The RISC-V CPU running on the FPGA will then decode those machine words and invoke the corresponding arithmetic hardware.

### Instruction Encoding

For the log2 and sqrt instructions, I used the RISC-V Reference Green Card to assign a funct7 field and an opcode to each one, making sure they do not conflict with any existing instruction definitions.

## Integer log2 and square root algorithm

### Log2

The integer log2(N) gives the minimum number of bits needed to represented N-1 distinct states. For a positive integer N, log2(N) is the smallest integer k such that 2k≥N. In hardware, an efficient way to compute this for a runtime input is to observe that for N>1, log2N=log2N-1+1. This means the problem reduces to the find the position of the highest set bit in N-1, which maps well to a leading-one detector or priority encoder in FPGA fabric.

Compared with a plain RV32I software implementation, a custom ceil(log2(N) instruction is much faster because the hardware can determine the highest set bit directly instead of iterating in a loop. In RV32I, a common method is to first compute N-1, then repeatedly shift right until the value becomes zero while counting the number of shifts. Since a 32-bit operand can require up to 32 shifts, the worst case is about 32 loop iterations. A simple hand-written RV32I loop usually needs about 4 to 6 instructions per iteration, such as a branch check, shift, increment, and jump back, so the total is roughly 130 to 200 cycles on a simple processor with little or no branch penalty hiding. Best case, for inputs like N≤1, is only a few cycles. By contrast, a custom FPGA hardware implementation based on a leading-one detector can usually produce the result in one combinational pass, so if integrated as a normal ALU-style custom instruction it can complete in about 1 cycle. So, in rough terms, the custom instruction reduces the operation from on the order of hundreds of cycles in software to about one cycle in hardware.

### Square Root

For the square root instruction, I looked into a few different hardware algorithms, including restoring, non-restoring, Newton-Raphson, and CORDIC. CORDIC is usually discussed more as a fixed-point iterative method, and it can be adapted for both log2 and sqrt, so it was part of the search as well. For this project though, I focused more on the standard integer square root approaches and Newton-Raphson.

Between the options, Newton-Raphson is attractive because it converges much faster. It can reach a good result in only a few iterations, which makes it appealing when latency matters. The nice part is that it is not necessarily much heavier in resource usage, but the implementation is still more involved because it depends on a good initial estimate and usually needs division or reciprocal-related operations in the iteration.

The restoring method is almost the opposite. Its biggest advantage is simplicity. The datapath and control are more straightforward, it is easier to understand, and it is usually easier to verify in hardware. That makes it a very practical choice for FPGA implementation. The downside is that it converges much more slowly, since it builds the result bit by bit and therefore takes more cycles.

So, the tradeoff is fairly clear. Restoring square root is simpler and more predictable, while Newton-Raphson is faster but more complex to implement cleanly. Right now, I am modeling and comparing these approaches in MATLAB, and I am also working on the FPGA implementation, simulation, and synthesis analysis to see how they compare in actual hardware.

# Identification of Resources

- Nexys4 FPGA Development Board

- MATLAB for algorithm modeling and Vivado for FPGA implementation and program

- IP to be developed

  - sqrt module

  - log2 module

  - custom instruction decode logic

  - control logic

  - memory-mapped interface for VGA display (optional)

- Monitor with VGA port and existing VGA controller from another JHU ECE class (optional)

# Updated Schedule for Project

| Module # | Objective |
| --- | --- |
| 10-11 | Integrate custom instructions into RV32IM pipeline. |
| 11-12 | Measure cycle counts and compares software vs hardware performance. |
| 12-13 | Optional VGA display integration. Complete testing, final report, and presentation preparation. |
| 14 | Documentation and Final Review |

# Expectation of Results

- Functional custom sqrt and log2 instructions integrated into RV32IM core

- Correct integer and fixed-point arithmetic verified against MATLAB reference

- Measurable cycle reduction

- Quantified performance improvement

- Optional VGA visualization of results and performance comparison

- FPGA resource utilization report by entity