# Computer Architecture Project Proposal

Custom log2 and sqrt ISA Extension for RV32IM with FPGA Acceleration and VGA Performance Visualization

# Scope of the Problem

In standard RV32IM architecture, integer square root (sqrt) and fixed-point logarithmic 2 are computed in software using iterative algorithms like Newton-Raphson. The iterative approach requires 80-150 clock cycles depending on the iteration count and specific multiplication and division algorithm in the hardware.

In real-world applications that require high throughput and low latency,  many heavy computations are completed in hardware using dedicated computational modules.

This project proposes:

- Extending a standard RV32IM core with custom hardware instructions:

  - sqrt – integer square root

  - log2 – fixed-point based-2 logarithm

- Implementing hardware accelerator

- Measuring and comparing computation cycle counts between

  - Software implementation (combination of RV32IM multiplication and division, and addition conditional branches

  - Custom ISA instruction implementation

- Optionally visualizing results and performance on a VGA monitor

# Relevance to Computer Architecture (RISC-V)

Custom ISA Extension

- Define new R-type instructions: sqrt and log2

- Use RISC-V reserved custom opcode space

- Demonstrates ISA-level architectural modification

Pipeline and Control Integration

- Add multi-cycle execution unit (like DIV in RV32M)

- Implement pipeline stall and writeback control

- Modify decode and execution stages

Datapath Extension

- Integrate hardware arithmetic accelerators into execution stage

- Route results back to register file

Memory System Interaction

- Store computation results into data memory

- Use memory-mapped region for VGA display

- Route data memory outputs to VGA controller

Memory-Mapped I/O and Peripheral Integration

- Demonstrate processor-to-peripheral communication

# Identification of Resources

- Nexys4 FPGA Development Board

- MATLAB for algorithm modeling and Vivado for FPGA implementation and program

- IP to be developed

  - sqrt module

  - log2 module

  - custom instruction decode logic

  - control logic

  - memory-mapped interface for VGA display

- Monitor with VGA port and existing VGA controller from another JHU ECE class

# Schedule for Project

| Module # | Objective |
| --- | --- |
| 6 | Finalize architecture, instruction encoding, and system design. |
| 7 & 8 | Model sqrt and log2 in MATLAB. Implement and verify integer sqrt accelerator. Identify which RISCV architecture to use (critical*) |
| 9 | Present progress and simulation results. Final opportunity to refine or modify project scope if necessary. |
| 10-11 | Implement log2 accelerator. Integrate custom instructions into RV32IM pipeline. |
| 11-12 | Measure cycle counts and compares software vs hardware performance. |
| 12-13 | Optional VGA display integration. Complete testing, final report, and presentation preparation. |
| 14 | Documentation and Final Review |

# Expectation of Results

- Functional custom sqrt and log2 instructions integrated into RV32IM core

- Correct integer and fixed-point arithmetic verified against MATLAB reference

- Measurable cycle reduction

- Quantified performance improvement

- Optional VGA visualization of results and performance comparison

- FPGA resource utilization report by entity